SYSTEM OVERVIEW

Satellite Detection → Processing & Modeling → Delivery & Alerts

DATA SOURCES & MODELS

Satellite Data Sources

- VIIRS: NOAA-20 & NPP satellites

- Sentinel-3 OLCI: ESA satellites

- Coverage: Ghana 0-20 nm offshore

- Update: Daily 06:00 UTC

Oceanographic model

- OpenDrift (Lagrangian model): An open-source particle-tracking framework used to simulate the drift and accumulation of Sargassum based on environmental forcing fields. It follows individual particles as they move with ocean currents and surface winds.

- Forcing Data:

  - RTOFS currents – provide 3D ocean current velocity fields.

  - GFS winds – supply surface wind vectors that influence drift at the ocean surface.

- Physics Implemented:

  - Advection: transport of Sargassum by ocean currents.

  - Wind drift (1%): windage effect representing the influence of surface winds on floating algae.

  - Diffusion: random turbulent spreading of particles to simulate small-scale variability.

DETECTION METHODS

- AFAI (Alternative Floating Algae Index):

  - Derived from VIIRS (Visible Infrared Imaging Radiometer Suite) data.

  - Utilizes Near-Infrared (NIR) bands to detect and quantify floating algae such as Sargassum.

  - Effective for distinguishing floating vegetation from surrounding seawater due to strong NIR reflectance.

- MCI (Maximum Chlorophyll Index):

  - Based on the OLCI (Ocean and Land Colour Instrument) sensor aboard Sentinel-3.

  - Sensitive to high chlorophyll concentrations typical of floating or surface vegetation.

  - Complements AFAI by enhancing detection in areas with variable lighting or water color conditions.

BACKEND PIPELINE

- GitHub Actions:

  - Automates the daily forecast generation workflow.

  - Runs scheduled jobs every day at 06:00 UTC for data ingestion, modeling, and web updates.

- Continuous Integration (CI/CD:

  - Automatically builds, tests, and deploys the system after each workflow run.

  - Ensures the web application and API remain production-ready and up-to-date.

- Quality Assurance Checks:

  - Includes validation of input satellite data and model outputs.

  - Detects anomalies or missing data and triggers fallback mechanisms.

  - Maintains high reliability and 99%+ system uptime.

TECHNICAL SPECIFICATIONS

- Forecast Range: 72 hours

- Spatial Resolution: 300m - 4km

- Update: Daily 06:00 UTC

- Coverage: Ghana EEZ (0-20 nm)

- Availability: 99% uptime

DATA PROCESSING

• Detection: AFAI / MCI spectral indices for Sargassum identification

• Model: OpenDrift v1.14.3 (Lagrangian drift simulation)

• Ocean Data: NOAA RTOFS (Real-Time Ocean Forecast System) currents

• Meteorological: NOAA GFS (Global Forecast System) winds

• Automation: GitHub Actions for daily forecast generation and QA checks

• Distribution: REST API + Interactive Web Application